# 9. Proposed Methodology

The proposed Multi-Level Optimization (MLO) architecture is organized into three vertically integrated levels. Each level receives signals from lower levels (actuation feedback) and sends commands downward, while simultaneously sharing information horizontally with adjacent levels through a shared context representation.

## 9(A) High-Level Optimization: Perception and Intention Recognition

### Sensing Architecture

The sensing subsystem comprises six degree-of-freedom IMUs (MEMS) placed at shank, thigh, and trunk segments (6 units per leg), capturing 3-axis linear accelerations a(t) ∈ ℝ³ and angular velocities ω(t) ∈ ℝ³ at 200 Hz, alongside plantar pressure insoles with 16 sensing elements per foot measuring spatiotemporal plantar pressure P(x,y,t) ∈ ℝ¹⁶ at 100 Hz. The fused sensor vector is:

x(t) = [a_L(t), ω_L(t), a_R(t), ω_R(t), a_T(t), ω_T(t), P_L(t), P_R(t)]ᵀ ∈ ℝᵈ, where d = 6×6 + 2×16 = 68.

### Fuzzy-CNN-GRU Architecture

The core perception model processes the multivariate time-series x(t) in a sliding window of W = 100 timesteps. A bank of K fuzzy kernel functions transforms raw data:

φ_k(x(t)) = exp(−‖x(t) − c_k‖² / 2σ_k²),  k = 1,…,K.

The fuzzy feature vector φ(t) = [φ₁(t),…,φ_K(t)]ᵀ is processed through N_c 1D convolutional layers extracting local temporal patterns, followed by two stacked GRU layers capturing long-range temporal dependencies with standard reset, update, and candidate gate equations. A softmax classification head produces posterior probabilities over M = 8 gait phases:

p(m|t) = exp(w_mᵀ h_T + b_m) / Σ_{j=1}^{M} exp(w_jᵀ h_T + b_j).

### Evolutionary Neural Architecture Search (Evo-NAS)

Rather than manual design, a multi-objective evolutionary NAS problem searches over architecture parameters (fuzzy kernels K, CNN layers, filter sizes, GRU units, dropout rates, window length). Each candidate α ∈ A is encoded as a chromosome and evaluated via bi-objective optimization:

min_{α∈A} F(α) = [L_val(α), C(α)],

where L_val is the validation cross-entropy loss and C(α) is computational cost (FLOPs). A Pareto-optimal set is obtained using NSGA-II selection. Continuous hyperparameters λ = (η, β₁, β₂, λ_wd, γ_lr) are refined via PSO with velocity and position updates:

v_i^(t+1) = ω v_i^(t) + c₁ r₁ (p_i − λ_i^(t)) + c₂ r₂ (g − λ_i^(t)),   λ_i^(t+1) = λ_i^(t) + v_i^(t+1).

### Intra-Subject Variability Modeling

Gait cycles are clustered into C variability modes using a Gaussian Mixture Model:

p(x) = Σ_{c=1}^{C} π_c  Ν(x; μ_c, Σ_c).

Parameters are estimated via EM on a per-subject basis, and online Bayesian updating propagates evidence from new gait cycles:

p(θ_c | x_{1:t}) ∝ p(x(t) | θ_c)  p(θ_c | x_{1:t−1}).

This feeds an anticipatory mechanism that pre-adjusts trajectory and controller parameters.

## 9(B) Mid-Level Optimization: Trajectory Generation

### NvGRF Modeling

Vertical ground reaction forces provide the canonical biomechanical gait signature. From plantar insoles, the discrete vGRF proxy is:

F_v(t) = Σ_{j=1}^{16} A_j · P_j(t),

normalized by body weight:

F̃_v(t) = F_v(t) / W_b.

A lightweight neural network f_ψ estimates F̃_v from IMU data when plantar sensors are unavailable, enabling graceful degradation. Sub-phases are detected via threshold crossings on F̃_v and its first derivative.

### Optimal Transport for Domain Adaptation

Let μ_s and μ_t be empirical gait distributions of source subject s and target subject t. The Wasserstein-2 distance is:

W₂(μ_s, μ_t) = [inf_{γ∈Γ(μ_s,μ_t)} ∫ ‖qˢ − qᵗ‖² dγ(qˢ, qᵗ)]^{1/2}.

The optimal transport map is obtained via the Sinkhorn algorithm:

P* = argmin_{P∈U(a,b)} ⟨C, P⟩ − ε H(P),

where C_{ij} = ‖q_iˢ − q_jᵗ‖² and H(P) = −Σ_{ij} P_{ij} log P_{ij}. For multi-subject personalization, the Wasserstein barycenter is computed:

μ̄ = argmin_μ Σ_{s=1}^{S} λ_s W₂²(μ, μ_s),

with weights λ_s set inversely proportional to anthropometric distance. This yields a personalized reference trajectory respecting the Riemannian geometry of probability distributions.

### Concept Drift Handling

A sliding-window Kolmogorov–Smirnov test monitors gait stationarity:

KS = sup_q |F_{1:t}(q) − F_{t−W:t}(q)|.

When the statistic exceeds threshold τ_KS, the transport map is re-estimated from recent data.

## 9(C) Low-Level Optimization: Robust Control

### System Model

The n-DOF exoskeleton Lagrangian dynamics are:

M(q) q̈ + C(q, q̇) q̇ + G(q) + τ_h + d(t) = τ_c.

Rewritten in ADRC canonical form per joint:

q̈ = (1/M₀) τ_c + f(q, q̇, d, t),

where f(·) is the total disturbance aggregating modeling errors, Coriolis/centrifugal effects, gravity, human torques, and external perturbations.

### Extended State Observer (ESO)

A third-order ESO estimates state and total disturbance with bandwidth parameters β₁, β₂, β₃ = 3ω_o, 3ω_o², ω_o³ placed at characteristic root −ω_o. Under bounded disturbance rate ḟ ≤ Δ_f, the ESO error is uniformly ultimately bounded:

lim sup_{t→∞} ‖e_o(t)‖ ≤ c Δ_f / ω_o.

### σ-Modified Adaptive ADRC (σ-AADRC)

For time-varying disturbance spectra, an adaptive law updates observer bandwidth:

ω̇_o = −γ_o e_oᵀ P_o B_o (q̂ − q) − σ_o ω_o.

The σ-modification term −σ_o ω_o prevents parameter drift, ensuring ω_o(t) ∈ [ω_min, ω_max]. The control law performs feedback linearization with disturbance compensation:

τ_c = M₀ [k_p (q_ref − q̂) + k_d (q̇_ref − q̂̇) + q̈_ref − f̂],

with bounded control modification:

τ_c^{sat} = τ_max · sign(τ_c)  if |τ_c| > τ_max,  else τ_c.

### Lyapunov Stability Analysis

Under conditions k_p > 0, k_d > 0, k_p k_d > Δ_f / f_max, the closed-loop system is uniformly ultimately bounded with bound:

‖[e_q, ė_q]ᵀ‖ ≤ ((k_p + k_d) / min(k_p, k_d)) · c Δ_f / ω_o + ε,

where ε → 0 as ω_o → ∞. The proof uses a composite Lyapunov function:

V(t) = e_cᵀ P_c e_c + e_oᵀ P_o e_o + (1/2γ_o)(ω_o − ω_o*)².

### Metaheuristic Online Parameter Tuning

The control parameter vector θ_c = [k_p, k_d, ω_o, σ_o, M₀]ᵀ is optimized online via Differential Evolution (population N_p = 10) on a 200 ms sliding window:

min_{θ_c} J(θ_c) = w₁‖e_q‖₂² + w₂‖ė_q‖₂² + w₃‖τ_c‖₁ + w₄ 𝟙(|τ_c| > τ_thresh),

penalizing torque magnitude and actuator saturation for safety and metabolic efficiency.

## 9(D) Work Plan and Activity Mapping

The three optimization levels are developed in parallel across Indian and French partner institutions, with defined milestones for integration testing at 12-month intervals, progressing from simulation to hardware-in-the-loop validation to clinical trials.